Team #09

Title: Project 5 Report

Course: Advanced Digital System Design (Fall 2022)

Names: Joseph Ayoka, Innocent Nortey, Kelvin Amuma, Renin Jose

REPORT

Design Summary:

In this Project, we will integrate an IP core of our own design to the Arm Cortex-A9 processor in the Cyclone V SoC on the DE1-SoC board. We will test our IP core using a Linux kernel module. The kernel module will provide a common programmable interface to userland software. Completion of this assignment will yield a starting roadmap to multiple fields such as SoC design, and heterogeneous computing.

Part 1

TOP level Module code

library ieee;

use ieee.std_logic_1164.all;

entity toplevel is

port (

clk_clk                         : in    std_logic                     := 'X';             -- clk

hps_io_hps_io_emac1_inst_TX_CLK : out   std_logic;                                        -- hps_io_emac1_inst_TX_CLK

hps_io_hps_io_emac1_inst_TXD0   : out   std_logic;                                        -- hps_io_emac1_inst_TXD0

hps_io_hps_io_emac1_inst_TXD1   : out   std_logic;                                        -- hps_io_emac1_inst_TXD1

hps_io_hps_io_emac1_inst_TXD2   : out   std_logic;                                        -- hps_io_emac1_inst_TXD2

hps_io_hps_io_emac1_inst_TXD3   : out   std_logic;                                        -- hps_io_emac1_inst_TXD3

hps_io_hps_io_emac1_inst_RXD0   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD0

hps_io_hps_io_emac1_inst_MDIO   : inout std_logic                     := 'X';             -- hps_io_emac1_inst_MDIO

hps_io_hps_io_emac1_inst_MDC    : out   std_logic;                                        -- hps_io_emac1_inst_MDC

hps_io_hps_io_emac1_inst_RX_CTL : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RX_CTL

hps_io_hps_io_emac1_inst_TX_CTL : out   std_logic;                                        -- hps_io_emac1_inst_TX_CTL

hps_io_hps_io_emac1_inst_RX_CLK : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RX_CLK

hps_io_hps_io_emac1_inst_RXD1   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD1

hps_io_hps_io_emac1_inst_RXD2   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD2

hps_io_hps_io_emac1_inst_RXD3   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD3

hps_io_hps_io_qspi_inst_IO0     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO0

hps_io_hps_io_qspi_inst_IO1     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO1

hps_io_hps_io_qspi_inst_IO2     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO2

hps_io_hps_io_qspi_inst_IO3     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO3

hps_io_hps_io_qspi_inst_SS0     : out   std_logic;                                        -- hps_io_qspi_inst_SS0

hps_io_hps_io_qspi_inst_CLK     : out   std_logic;                                        -- hps_io_qspi_inst_CLK

hps_io_hps_io_sdio_inst_CMD     : inout std_logic                     := 'X';             -- hps_io_sdio_inst_CMD

hps_io_hps_io_sdio_inst_D0      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D0

hps_io_hps_io_sdio_inst_D1      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D1

hps_io_hps_io_sdio_inst_CLK     : out   std_logic;                                        -- hps_io_sdio_inst_CLK

hps_io_hps_io_sdio_inst_D2      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D2

hps_io_hps_io_sdio_inst_D3      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D3

hps_io_hps_io_usb1_inst_D0      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D0

hps_io_hps_io_usb1_inst_D1      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D1

hps_io_hps_io_usb1_inst_D2      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D2

hps_io_hps_io_usb1_inst_D3      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D3

hps_io_hps_io_usb1_inst_D4      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D4

hps_io_hps_io_usb1_inst_D5      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D5

hps_io_hps_io_usb1_inst_D6      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D6

hps_io_hps_io_usb1_inst_D7      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D7

hps_io_hps_io_usb1_inst_CLK     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_CLK

hps_io_hps_io_usb1_inst_STP     : out   std_logic;                                        -- hps_io_usb1_inst_STP

hps_io_hps_io_usb1_inst_DIR     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_DIR

hps_io_hps_io_usb1_inst_NXT     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_NXT

hps_io_hps_io_spim1_inst_CLK    : out   std_logic;                                        -- hps_io_spim1_inst_CLK

hps_io_hps_io_spim1_inst_MOSI   : out   std_logic;                                        -- hps_io_spim1_inst_MOSI

hps_io_hps_io_spim1_inst_MISO   : in    std_logic                     := 'X';             -- hps_io_spim1_inst_MISO

hps_io_hps_io_spim1_inst_SS0    : out   std_logic;                                        -- hps_io_spim1_inst_SS0

hps_io_hps_io_uart0_inst_RX     : in    std_logic                     := 'X';             -- hps_io_uart0_inst_RX

hps_io_hps_io_uart0_inst_TX     : out   std_logic;                                        -- hps_io_uart0_inst_TX

hps_io_hps_io_i2c0_inst_SDA     : inout std_logic                     := 'X';             -- hps_io_i2c0_inst_SDA

hps_io_hps_io_i2c0_inst_SCL     : inout std_logic                     := 'X';             -- hps_io_i2c0_inst_SCL

hps_io_hps_io_i2c1_inst_SDA     : inout std_logic                     := 'X';             -- hps_io_i2c1_inst_SDA

hps_io_hps_io_i2c1_inst_SCL     : inout std_logic                     := 'X';             -- hps_io_i2c1_inst_SCL

memory_mem_a                    : out   std_logic_vector(14 downto 0);                    -- mem_a

memory_mem_ba                   : out   std_logic_vector(2 downto 0);                     -- mem_ba

memory_mem_ck                   : out   std_logic;                                        -- mem_ck

memory_mem_ck_n                 : out   std_logic;                                        -- mem_ck_n

memory_mem_cke                  : out   std_logic;                                        -- mem_cke

memory_mem_cs_n                 : out   std_logic;                                        -- mem_cs_n

memory_mem_ras_n                : out   std_logic;                                        -- mem_ras_n

memory_mem_cas_n                : out   std_logic;                                        -- mem_cas_n

memory_mem_we_n                 : out   std_logic;                                        -- mem_we_n

memory_mem_reset_n              : out   std_logic;                                        -- mem_reset_n

memory_mem_dq                   : inout std_logic_vector(31 downto 0) := (others => 'X'); -- mem_dq

memory_mem_dqs                  : inout std_logic_vector(3 downto 0)  := (others => 'X'); -- mem_dqs

memory_mem_dqs_n                : inout std_logic_vector(3 downto 0)  := (others => 'X'); -- mem_dqs_n

memory_mem_odt                  : out   std_logic;                                        -- mem_odt

memory_mem_dm                   : out   std_logic_vector(3 downto 0);                     -- mem_dm

memory_oct_rzqin                : in    std_logic                     := 'X';             -- oct_rzqin

digits		                    : out   std_logic_vector(41 downto 0)                     -- export

);

end entity toplevel;

architecture rtl of toplevel is

component soc_calc_base is

port (

clk_clk                         : in    std_logic                     := 'X';             -- clk

hps_io_hps_io_emac1_inst_TX_CLK : out   std_logic;                                        -- hps_io_emac1_inst_TX_CLK

hps_io_hps_io_emac1_inst_TXD0   : out   std_logic;                                        -- hps_io_emac1_inst_TXD0

hps_io_hps_io_emac1_inst_TXD1   : out   std_logic;                                        -- hps_io_emac1_inst_TXD1

hps_io_hps_io_emac1_inst_TXD2   : out   std_logic;                                        -- hps_io_emac1_inst_TXD2

hps_io_hps_io_emac1_inst_TXD3   : out   std_logic;                                        -- hps_io_emac1_inst_TXD3

hps_io_hps_io_emac1_inst_RXD0   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD0

hps_io_hps_io_emac1_inst_MDIO   : inout std_logic                     := 'X';             -- hps_io_emac1_inst_MDIO

hps_io_hps_io_emac1_inst_MDC    : out   std_logic;                                        -- hps_io_emac1_inst_MDC

hps_io_hps_io_emac1_inst_RX_CTL : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RX_CTL

hps_io_hps_io_emac1_inst_TX_CTL : out   std_logic;                                        -- hps_io_emac1_inst_TX_CTL

hps_io_hps_io_emac1_inst_RX_CLK : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RX_CLK

hps_io_hps_io_emac1_inst_RXD1   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD1

hps_io_hps_io_emac1_inst_RXD2   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD2

hps_io_hps_io_emac1_inst_RXD3   : in    std_logic                     := 'X';             -- hps_io_emac1_inst_RXD3

hps_io_hps_io_qspi_inst_IO0     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO0

hps_io_hps_io_qspi_inst_IO1     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO1

hps_io_hps_io_qspi_inst_IO2     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO2

hps_io_hps_io_qspi_inst_IO3     : inout std_logic                     := 'X';             -- hps_io_qspi_inst_IO3

hps_io_hps_io_qspi_inst_SS0     : out   std_logic;                                        -- hps_io_qspi_inst_SS0

hps_io_hps_io_qspi_inst_CLK     : out   std_logic;                                        -- hps_io_qspi_inst_CLK

hps_io_hps_io_sdio_inst_CMD     : inout std_logic                     := 'X';             -- hps_io_sdio_inst_CMD

hps_io_hps_io_sdio_inst_D0      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D0

hps_io_hps_io_sdio_inst_D1      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D1

hps_io_hps_io_sdio_inst_CLK     : out   std_logic;                                        -- hps_io_sdio_inst_CLK

hps_io_hps_io_sdio_inst_D2      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D2

hps_io_hps_io_sdio_inst_D3      : inout std_logic                     := 'X';             -- hps_io_sdio_inst_D3

hps_io_hps_io_usb1_inst_D0      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D0

hps_io_hps_io_usb1_inst_D1      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D1

hps_io_hps_io_usb1_inst_D2      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D2

hps_io_hps_io_usb1_inst_D3      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D3

hps_io_hps_io_usb1_inst_D4      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D4

hps_io_hps_io_usb1_inst_D5      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D5

hps_io_hps_io_usb1_inst_D6      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D6

hps_io_hps_io_usb1_inst_D7      : inout std_logic                     := 'X';             -- hps_io_usb1_inst_D7

hps_io_hps_io_usb1_inst_CLK     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_CLK

hps_io_hps_io_usb1_inst_STP     : out   std_logic;                                        -- hps_io_usb1_inst_STP

hps_io_hps_io_usb1_inst_DIR     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_DIR

hps_io_hps_io_usb1_inst_NXT     : in    std_logic                     := 'X';             -- hps_io_usb1_inst_NXT

hps_io_hps_io_spim1_inst_CLK    : out   std_logic;                                        -- hps_io_spim1_inst_CLK

hps_io_hps_io_spim1_inst_MOSI   : out   std_logic;                                        -- hps_io_spim1_inst_MOSI

hps_io_hps_io_spim1_inst_MISO   : in    std_logic                     := 'X';             -- hps_io_spim1_inst_MISO

hps_io_hps_io_spim1_inst_SS0    : out   std_logic;                                        -- hps_io_spim1_inst_SS0

hps_io_hps_io_uart0_inst_RX     : in    std_logic                     := 'X';             -- hps_io_uart0_inst_RX

hps_io_hps_io_uart0_inst_TX     : out   std_logic;                                        -- hps_io_uart0_inst_TX

hps_io_hps_io_i2c0_inst_SDA     : inout std_logic                     := 'X';             -- hps_io_i2c0_inst_SDA

hps_io_hps_io_i2c0_inst_SCL     : inout std_logic                     := 'X';             -- hps_io_i2c0_inst_SCL

hps_io_hps_io_i2c1_inst_SDA     : inout std_logic                     := 'X';             -- hps_io_i2c1_inst_SDA

hps_io_hps_io_i2c1_inst_SCL     : inout std_logic                     := 'X';             -- hps_io_i2c1_inst_SCL

memory_mem_a                    : out   std_logic_vector(14 downto 0);                    -- mem_a

memory_mem_ba                   : out   std_logic_vector(2 downto 0);                     -- mem_ba

memory_mem_ck                   : out   std_logic;                                        -- mem_ck

memory_mem_ck_n                 : out   std_logic;                                        -- mem_ck_n

memory_mem_cke                  : out   std_logic;                                        -- mem_cke

memory_mem_cs_n                 : out   std_logic;                                        -- mem_cs_n

memory_mem_ras_n                : out   std_logic;                                        -- mem_ras_n

memory_mem_cas_n                : out   std_logic;                                        -- mem_cas_n

memory_mem_we_n                 : out   std_logic;                                        -- mem_we_n

memory_mem_reset_n              : out   std_logic;                                        -- mem_reset_n

memory_mem_dq                   : inout std_logic_vector(31 downto 0) := (others => 'X'); -- mem_dq

memory_mem_dqs                  : inout std_logic_vector(3 downto 0)  := (others => 'X'); -- mem_dqs

memory_mem_dqs_n                : inout std_logic_vector(3 downto 0)  := (others => 'X'); -- mem_dqs_n

memory_mem_odt                  : out   std_logic;                                        -- mem_odt

memory_mem_dm                   : out   std_logic_vector(3 downto 0);                     -- mem_dm

memory_oct_rzqin                : in    std_logic                     := 'X';             -- oct_rzqin

reset_reset_n                   : in    std_logic                     := 'X';             -- reset_n

digits_export                   : out   std_logic_vector(41 downto 0)                     -- export

);

end component soc_calc_base;

begin

u0 : component soc_calc_base

port map (

clk_clk                         => clk_clk,                         --    clk.clk

hps_io_hps_io_emac1_inst_TX_CLK => hps_io_hps_io_emac1_inst_TX_CLK, -- hps_io.hps_io_emac1_inst_TX_CLK

hps_io_hps_io_emac1_inst_TXD0   => hps_io_hps_io_emac1_inst_TXD0,   --       .hps_io_emac1_inst_TXD0

hps_io_hps_io_emac1_inst_TXD1   => hps_io_hps_io_emac1_inst_TXD1,   --       .hps_io_emac1_inst_TXD1

hps_io_hps_io_emac1_inst_TXD2   => hps_io_hps_io_emac1_inst_TXD2,   --       .hps_io_emac1_inst_TXD2

hps_io_hps_io_emac1_inst_TXD3   => hps_io_hps_io_emac1_inst_TXD3,   --       .hps_io_emac1_inst_TXD3

hps_io_hps_io_emac1_inst_RXD0   => hps_io_hps_io_emac1_inst_RXD0,   --       .hps_io_emac1_inst_RXD0

hps_io_hps_io_emac1_inst_MDIO   => hps_io_hps_io_emac1_inst_MDIO,   --       .hps_io_emac1_inst_MDIO

hps_io_hps_io_emac1_inst_MDC    => hps_io_hps_io_emac1_inst_MDC,    --       .hps_io_emac1_inst_MDC

hps_io_hps_io_emac1_inst_RX_CTL => hps_io_hps_io_emac1_inst_RX_CTL, --       .hps_io_emac1_inst_RX_CTL

hps_io_hps_io_emac1_inst_TX_CTL => hps_io_hps_io_emac1_inst_TX_CTL, --       .hps_io_emac1_inst_TX_CTL

hps_io_hps_io_emac1_inst_RX_CLK => hps_io_hps_io_emac1_inst_RX_CLK, --       .hps_io_emac1_inst_RX_CLK

hps_io_hps_io_emac1_inst_RXD1   => hps_io_hps_io_emac1_inst_RXD1,   --       .hps_io_emac1_inst_RXD1

hps_io_hps_io_emac1_inst_RXD2   => hps_io_hps_io_emac1_inst_RXD2,   --       .hps_io_emac1_inst_RXD2

hps_io_hps_io_emac1_inst_RXD3   => hps_io_hps_io_emac1_inst_RXD3,   --       .hps_io_emac1_inst_RXD3

hps_io_hps_io_qspi_inst_IO0     => hps_io_hps_io_qspi_inst_IO0,     --       .hps_io_qspi_inst_IO0

hps_io_hps_io_qspi_inst_IO1     => hps_io_hps_io_qspi_inst_IO1,     --       .hps_io_qspi_inst_IO1

hps_io_hps_io_qspi_inst_IO2     => hps_io_hps_io_qspi_inst_IO2,     --       .hps_io_qspi_inst_IO2

hps_io_hps_io_qspi_inst_IO3     => hps_io_hps_io_qspi_inst_IO3,     --       .hps_io_qspi_inst_IO3

hps_io_hps_io_qspi_inst_SS0     => hps_io_hps_io_qspi_inst_SS0,     --       .hps_io_qspi_inst_SS0

hps_io_hps_io_qspi_inst_CLK     => hps_io_hps_io_qspi_inst_CLK,     --       .hps_io_qspi_inst_CLK

hps_io_hps_io_sdio_inst_CMD     => hps_io_hps_io_sdio_inst_CMD,     --       .hps_io_sdio_inst_CMD

hps_io_hps_io_sdio_inst_D0      => hps_io_hps_io_sdio_inst_D0,      --       .hps_io_sdio_inst_D0

hps_io_hps_io_sdio_inst_D1      => hps_io_hps_io_sdio_inst_D1,      --       .hps_io_sdio_inst_D1

hps_io_hps_io_sdio_inst_CLK     => hps_io_hps_io_sdio_inst_CLK,     --       .hps_io_sdio_inst_CLK

hps_io_hps_io_sdio_inst_D2      => hps_io_hps_io_sdio_inst_D2,      --       .hps_io_sdio_inst_D2

hps_io_hps_io_sdio_inst_D3      => hps_io_hps_io_sdio_inst_D3,      --       .hps_io_sdio_inst_D3

hps_io_hps_io_usb1_inst_D0      => hps_io_hps_io_usb1_inst_D0,      --       .hps_io_usb1_inst_D0

hps_io_hps_io_usb1_inst_D1      => hps_io_hps_io_usb1_inst_D1,      --       .hps_io_usb1_inst_D1

hps_io_hps_io_usb1_inst_D2      => hps_io_hps_io_usb1_inst_D2,      --       .hps_io_usb1_inst_D2

hps_io_hps_io_usb1_inst_D3      => hps_io_hps_io_usb1_inst_D3,      --       .hps_io_usb1_inst_D3

hps_io_hps_io_usb1_inst_D4      => hps_io_hps_io_usb1_inst_D4,      --       .hps_io_usb1_inst_D4

hps_io_hps_io_usb1_inst_D5      => hps_io_hps_io_usb1_inst_D5,      --       .hps_io_usb1_inst_D5

hps_io_hps_io_usb1_inst_D6      => hps_io_hps_io_usb1_inst_D6,      --       .hps_io_usb1_inst_D6

hps_io_hps_io_usb1_inst_D7      => hps_io_hps_io_usb1_inst_D7,      --       .hps_io_usb1_inst_D7

hps_io_hps_io_usb1_inst_CLK     => hps_io_hps_io_usb1_inst_CLK,     --       .hps_io_usb1_inst_CLK

hps_io_hps_io_usb1_inst_STP     => hps_io_hps_io_usb1_inst_STP,     --       .hps_io_usb1_inst_STP

hps_io_hps_io_usb1_inst_DIR     => hps_io_hps_io_usb1_inst_DIR,     --       .hps_io_usb1_inst_DIR

hps_io_hps_io_usb1_inst_NXT     => hps_io_hps_io_usb1_inst_NXT,     --       .hps_io_usb1_inst_NXT

hps_io_hps_io_spim1_inst_CLK    => hps_io_hps_io_spim1_inst_CLK,    --       .hps_io_spim1_inst_CLK

hps_io_hps_io_spim1_inst_MOSI   => hps_io_hps_io_spim1_inst_MOSI,   --       .hps_io_spim1_inst_MOSI

hps_io_hps_io_spim1_inst_MISO   => hps_io_hps_io_spim1_inst_MISO,   --       .hps_io_spim1_inst_MISO

hps_io_hps_io_spim1_inst_SS0    => hps_io_hps_io_spim1_inst_SS0,    --       .hps_io_spim1_inst_SS0

hps_io_hps_io_uart0_inst_RX     => hps_io_hps_io_uart0_inst_RX,     --       .hps_io_uart0_inst_RX

hps_io_hps_io_uart0_inst_TX     => hps_io_hps_io_uart0_inst_TX,     --       .hps_io_uart0_inst_TX

hps_io_hps_io_i2c0_inst_SDA     => hps_io_hps_io_i2c0_inst_SDA,     --       .hps_io_i2c0_inst_SDA

hps_io_hps_io_i2c0_inst_SCL     => hps_io_hps_io_i2c0_inst_SCL,     --       .hps_io_i2c0_inst_SCL

hps_io_hps_io_i2c1_inst_SDA     => hps_io_hps_io_i2c1_inst_SDA,     --       .hps_io_i2c1_inst_SDA

hps_io_hps_io_i2c1_inst_SCL     => hps_io_hps_io_i2c1_inst_SCL,     --       .hps_io_i2c1_inst_SCL

memory_mem_a                    => memory_mem_a,                    -- memory.mem_a

memory_mem_ba                   => memory_mem_ba,                   --       .mem_ba

memory_mem_ck                   => memory_mem_ck,                   --       .mem_ck

memory_mem_ck_n                 => memory_mem_ck_n,                 --       .mem_ck_n

memory_mem_cke                  => memory_mem_cke,                  --       .mem_cke

memory_mem_cs_n                 => memory_mem_cs_n,                 --       .mem_cs_n

memory_mem_ras_n                => memory_mem_ras_n,                --       .mem_ras_n

memory_mem_cas_n                => memory_mem_cas_n,                --       .mem_cas_n

memory_mem_we_n                 => memory_mem_we_n,                 --       .mem_we_n

memory_mem_reset_n              => memory_mem_reset_n,              --       .mem_reset_n

memory_mem_dq                   => memory_mem_dq,                   --       .mem_dq

memory_mem_dqs                  => memory_mem_dqs,                  --       .mem_dqs

memory_mem_dqs_n                => memory_mem_dqs_n,                --       .mem_dqs_n

memory_mem_odt                  => memory_mem_odt,                  --       .mem_odt

memory_mem_dm                   => memory_mem_dm,                   --       .mem_dm

memory_oct_rzqin                => memory_oct_rzqin,                --       .oct_rzqin

reset_reset_n                   => '1',                   								--  reset.reset_n

digits_export                   => digits								                  -- digits.export

);

end architecture rtl;

Simulation